Notes from the Editor:

Time for the Fall shows, especially the big AACA meet at Hershey Pennsylvania on October 10 through 13.  If any members are going, your editor (Jan Eyerman) has a flea market spot in the Chocolate field at Aisle I, spot 8.  I will be with the Crosley Club (an old friend of mine is a big Crosley fan).  So if you are at the meet, drop by.  I won't be selling any Hillman stuff (I keep that for club members) but I will have plenty of "junk" or should that be "junque".

This has been a bad year for car shows, those shows that weren't rained out were so hot as to make me leave early or not go at all.  Hopefully September and October will be better.  Hope you enjoy the articles in this issue of Melodies!

Nail Polish?

A very useful addition to your toolbox are a couple of bottles of nail polish.  They do not need to be expensive, in fact the cheapest stuff you can find will do fine.  Garage sales could be a good source as are "Dollar" stores.  If you can get some from someone who uses it, an obsolete or unwanted color will be OK too.  I picked up a bottle of a silver white color and a bright orange-red at a garage sale for ten cents each.

Nail polish has many uses.  It is water-proof, it comes with it's own brush/applicator, is a bright color and dries quickly.  I use it to mark the timing mark on my car's front pulley and to lightly paint the "micam" brake adjusters on my Series IIIC Minx.  All Hillmans from 1948 through 1963 (and later Husky's) uses Lockheed brakes that are adjusted through a hole in the brake drum (and if the road wheel is installed correctly, through a hole in the road wheel) via what appears to be a screw head. You simply insert a flat bladed screw driver and turn clockwise until you can't turn it anymore and then back off "one click".  The problem is finding the screw head! Painting the screw head lightly with red or red-orange nail polish makes it much easier to find when peering through that small hole.

Radios

The name of this newsletter commemorates the first car to be equipped with a radio, the 1934 "Melody Minx".  Rootes offered a special model of the Hillman with a radio built into the dashboard, along with a built in antenna.  Quite an original idea for 1934!  However, after the original Melody Minx, radios became an add on and in the USA things became a bit difficult.

I am going to limit this discussion to "AM" radios as that was the most common and popular type of car radio up though the late 1960's. Let's start with something called "frequencies".  The USA (and Canada) use different frequencies for their radios then England and Australia (and for that matter many other countries), so a British or Australian radio won't pick up many, if any, US radio stations.  It does appear that New Zealand used and still uses the same radio frequencies as the USA, so a New Zealand radio might work.

Next problem is voltage.  Back in the forties and fifties, American cars had six volt electrical systems while British cars (including Hillmans) had twelve volt electrical systems.  The translation is that the electrical system will burn out an American car radio from that era if installed in a Hillman.

Fortunately American cars went to twelve volts in the mid to late fifties, so later radios don't have that problem.  However, there is still another problem and it is called polarity.  British cars used Lucas electrical systems and Joseph Lucas was a believer in the "good earth" so he designed his electrical systems as Positive Earth, or as it is known in America as Positive Ground.  Unfortunately, ALL American twelve systems were and are Negative Ground.

If you connect a Negative Ground radio into a Positive Ground car, bad things will happen and the radio will be destroyed, it could also set the wiring on fire.

So I far I have explained the problems. Fortunately there are indeed solutions and I will explain them. Several radio manufacturers in both England and the USA recognized both the problem and the opportunity it provided them to sell car radios for British cars in the USA.  In England a company called "His Master's Voice" made positive ground radios that could receive American Radio stations (in the 660-1500 Kilocycle range) and these could be purchased from many British car dealers in the USA.

In the USA Motorola initially came out with a unique line of radios called "floating ground" that could be connected to both positive ground and negative ground cars.  I happen to have one of these and it works quite well in either connection.  Motorola later came out with a car radio that had sort of switch on the top of the radio that allowed the selection of either positive ground or negative ground.  This also worked quite well.  In addition, this radio had a built in speaker, making quite easy to install.

There were two versions of this radio.  A basic unit with just a knob controlling the station selector and a "Deluxe" version that had mechanical push buttons that allowed you to pre-set five stations in addition to the tuning knob.

Installation of any of these radios (and a number of other brands that offered either "positive ground only" or switchable ground) is quite straight forward.  You can connect a wire from the "accessory" connector on the ignition switch, run through an "in-line" fuse holder (use a 3 amp or 5 amp fuse) and connect to the single wire coming from the radio. Then bolt the radio in any convenient spot in or under the dash. On my Series IIIC I fabricated a mount that slides into the ashtray slot.  You will also need a ground wire that goes from the radio body to a good ground behind the dash.  The Mark cars and the Series I & II Minx had a special slot for a radio built into the dash.  You will have to get a bit creative to fit the radio in, but it can be done.  Series III and later Minx DeLuxes along with all of the Super Minxes required a special radio mount, these are very rare and expensive today.  My first '59 Minx had one and I greatly regret selling with the car back in 1965!  No other Minx I have owned has had one! Minx Specials and Huskys require total fabrication of a mount.  Below is a picture of a mount I fabricated for my 1959 Minx Special for a NIB Motorola dual ground radio I bought on e-bay for $30.

Motorola Radio installed in a 1959 Minx Special

Motorola radio installed on right side of dash of 1959 Minx Special

All you need then is an antenna.  You can buy "period" style telescoping antenna's on the internet or at a good auto parts store.  Then drill a large hole in a fender and feed the wire through to behind the dash and plug it into the antenna hole in the radio.  Then if you are within reasonable range of radio station, turn it on and listen to AM radio.

If you get lots of "crackle" and static in the radio when the engine is running, the problem is either your generator or ignition.  You will need two "Radio Surpression Capacitors" (or as it is sometimes called, a Condensor). One gets hooked onto the "D" connector on your voltage regulator and the other on the coil. Both should be connected to a good body ground.  This usually works in eliminating the crackle and most static.  Unfortunately some cars are much harder to "quiet" down.  One solution is to replace your generator's voltage regulator with a new electronic one.

I have not spoken about newer "FM" radios as these began getting popular in the late sixties and early seventies.  For example, the Cricket came only with an AM radio from Chrysler, no FM was available.  You can modify the dash to accept a FM radio which was done to my '73 Cricket before I got it (it also had a CD player).  Almost all FM or AM-FM radios I have seen are negative ground.  This more or less precludes their use in any of the Pre-Arrow cars unless the polarity of the electrical system has been changed.

An AM-FM radio can be installed in an Arrow as this is a simple under-dash mount and the electrical system is already negative ground.  Dual speakers can be mounted at either end of the parcel shelf and you can enjoy stereo FM while driving.

I am currently working a small, short range AM transmitter that can be hooked up to a portable CD player to allow me to play vintage music from CD's on my '62 Minx's AM radio.  When I finish that project, if it is successful, I will write an article about it for Melodies.

Transmission and Rear End Lube:

In the previous issue I spoke about Engine oil and filters, etc.  In this issue we move to the Transmission and rear end, plus a little bit about the later steering boxes.  Let's start with the transmission.  The correct lubricant is 20 weight motor oil. This is somewhat hard to find but you can use a straight, non-detergent 30 weight or 10W30 motor oil.  I have been trying 20W50 in my Minx Series IIIC and it seems to be doing OK.  There is a large drain plug conveniently located at the center, bottom of the transmission.  Put a large wrench on it and drain out the oil into a pan.  The transmission holds about 2 quarts of oil.  Once the oil is drained there are two possible places to fill it.  On the older cars with the remote floorshift or a column shift there is a rubber plug on top of the transmission tunnel that give access to a dip-stick.  The dip-stick hole is the filler.  On cars with the older, long direct acting floor shift, the large floor shift rubber boot must be removed to expose the dipstick.  On still later model cars there is an access panel under the transmission hump carpet on the passenger's side that allows you to access a plug on the side of the transmission.  Fill to the level of the plug hole.  This whole operation requires you to be careful as you can get motor oil on the floor or carpet.

On Arrows and Crickets you must get under the car to access the filler plug on the right side of the transmission. I use a long, snaking hose from a funnel in the engine compartment.  While this is a bit of a pain in the neck, it does keep the inside of the car clean.

The rear end is much more straight forward.  In the center of the rear axle is a drain plug, un-screw it and out will come out about a quart of rear end oil. There is a filler plug on the top right side of the "extension" of the rear axle towards the driveshaft.  It is square sided so you must use an open end wrench.  Clean the area around it thoroughly before removing it to keep dirt out of the rear axle. Refill with EP-90.  When the oil is up to the bottom of the filler hole, it is full.  BE CAREFUL NOT TO OVERFILL or you will get oil out of the rear axle seals and onto the brakes.

On later cars with recirculating ball steering (Series III through Series VI and Arrows) the steering box is filled with EP90.  There is a rubber plug on the top cover.  This will not be easy to find if your car has a column shift.  Again, clean the entire area around the plug and remove using a flat bladed screw driver.  The steering box cannot be drained and holds about ½  pint of oil.  The Mark cars and the Series I & II Minx have a grease fitting so all you need to do is to hit it with the grease gun.

The steering box on the Cricket and the Imp is a rack and pinion unit.  These steering boxes do not have any filler plugs, but do take EP-90.  To fill them you must first jack up one side of the car, I do the right side so that the car is tilted to one side.  Then remove a rubber "boot" on the steering rack.  Pour in EP90 until it is nearly full. Then replace the boot, use a hose clamp to hold it in place if necessary and lower the car back onto the ground.  Kind of a strange way to do it, but that appears to be the only way to get lubricant into the mechanism.  As a by the way, there are "universal" boots available at auto parts stores in case the boots on your car bad or damaged.

End of Citreon-Peugeot?

The current economic problems in Europe are seriously impacting PSA Peugeot-Citreon, the company that bought Chrysler UK (the old Rootes company) from Chrysler back in 1978.  PSA Peugeot -Citreon- renamed all of the cars "Talbots" after the purchase and gradually phased out production, the last Talbot cars being built and sold in 1987 and the last trucks in 1992.  This spring it was announced that…  "On 29 February 2012, PSA formally announced the creation of a major alliance with General Motors (GM), as part of which GM became PSA's second-largest shareholder, after the Peugeot family, with a holding of 7%. The alliance is intended to enable $2 billion per year of cost savings through platform sharing, common purchasing and other economies of scale."

"In July 2012, a union official said that PSA Peugeot Citroen will cut as 10 percent (8,000-10,000) of its French workforce of 100,356 employees on permanent and temporary contract. The jobs cut is more than previously announced."

However, PSA Peugeot Citreon needs additional working capital and there is a good question as to whether General Motors can support PSA.  Especially given that General Motors' Opel subsidiary is losing large sums of money every year.  I will be following the fate of the last bit of Rootes that still exists over the next few months and keep everyone posted!

Cricket Parts:

Just a note for members who own Plymouth Crickets, periodically chromed plastic "Cricket" nameplates appear on e-bay.  These are not actually from the Chrysler UK (ex-Rootes) cars, but from a short lived Mitsubishi based car sold in Canada as a replacement for the Cricket in 1973.  For 1971 and 1972 the Canadian version of the Cricket was the same as the US version (a variation of the Hillman Avenger), in 1973 the US version was still based on the Hillman Avenger, but the Canadian car was based on the Mitsubishi "Colt".  Be careful when buying parts for a Cricket from Canada as they could be for the Japanese car and not the US car.  However, the plastic Chromed name plates will work on your car and will look and last better then the decal.  As a by the way, 40 year old NOS decals are probably no longer useful and will fall apart if you try to apply them.

Saying that, there are many Cricket parts that will fit an Arrow or later Minx.  Check out especially "Rock Auto Parts" on line.  They have excellent prices and carry a surprising number of parts for the Minx, Sunbeam Arrow and Cricket.  A good example being Universal joints, the ones for a Cricket will fit all Hillmans back to the Series I.

2013 Calendars:

For the past 5 years or so a Hillman collector from Australia has been having Hillman Calendars made by an on-line publishing company called "Lulu.com".  The 2012 calendars should be available in late 2012 and will cost about $12 plus shipping.  I have a nice collection of the past calendars and the big 8 ½ x 11 color pictures of Hillmans from around the world are great to look at!